A Comparison of Feature Extraction Methods for EEG Signals1

A. Moura, S. Lopez, I. Obeid and J. Picone

The Neural Engineering Data Consortium

Temple University

andersongmoura@gmail.com, iobeid@temple.edu, joseph.picone@gmail.com

Feature extraction for automatic interpretation of EEGs has been extensively studied. A number of commercial approaches use exotic feature sets such as wavelets or nonlinear statistical measures such as fractal dimension. These choices of features were the results of evaluations and optimizations conducted on small research databases often collected under very controlled conditions. These approaches have not been extensively evaluated on big data or clinical applications using state of the art machine learning technology. Therefore, in this study, we compare performance of a number of standard feature extraction techniques on the publicly available TUH EEG Corpus using a state of the art classification system.

The TUH EEG Corpus is the largest publicly available corpus of EEG data. It comprises over 28,000 sessions collected from 2002-2015 at Temple University Hospital. It is entirely composed of clinical data, which means the data is representative of all the problems typically encountered in clinical settings, such as patient movement, artifacts due to eye blinks, talking, etc. Such data poses a much different challenge for machine learning systems since rejection of background noise becomes a critical issue.

The classification system used for this study, known as AutoEEGTM, automatically recognizes specific events in the EEG data and generates annotations. AutoEEGTM is based on a hidden Markov model (HMM) approach to modeling the temporal evolution of the spectrum. A maximum likelihood (ML) approach is used to train standard three-state HMMs consisting of 8 Gaussian mixtures per state and diagonal covariance matrices.

The system detects three events of clinical interest (PLED, GPED and SPSW) and three events used to model background noise (ARTF, EYEM and BCKG). The current system uses an enhanced feature extraction approach based on Mel Frequency Cepstral Coefficients (MFCC’s) together with differential energy, first and second derivatives. In this study, we evaluated 15 features, by augmenting the standard feature vector with one additional feature. These were evaluated on a subset of the TUH EEG Corpus. This subset was designed to give rapid turnaround on experiments, and to ensure these results will correlate well with results on the full dataset. This particular set of 15 features was chosen based on analysis of historically significant publications in the field.

- Research reported in this publication was supported by the National Human Genome Research Institute of the National Institutes of Health under Award Number U01HG008468.

- This research was also supported by the Brazil Scientific Mobility Program (BSMP) and the Institute of International Education (IIE).

In general, our findings indicate that any of these features individually influence performance very little. This contradicts findings previously published, but was not unexpected. Clinical data is extremely challenging and quite different from most published EEG corpora. Maximum Fractal Length (MFL) provided the greatest reduction in error rate, though the improvements were not statistically significant.

The MFCC approach has been in use for speech recognition applications for several decades and is known to provide a robust characterization of the temporal and spectral properties of the signal. In related work, we demonstrate that wavelets, which are often proposed as an alternative to MFCCs, also provide no gain in performance. Though literature suggests that EEG signals can be viewed as chaotic time series with significant amounts of nonlinearities, the features we investigated, which are designed to characterize such properties, add little value to our standard feature extraction approaches. Future research will be focused on better time-frequency representations of the signal based on correlation and coherence.